EBU6018 Advanced Transform Methods

EBU5303 Multimedia Fundamentals

Lab 2 – 16 November 2022

# Introduction

This lab is common to EBU6018 and EBU5303.

In Part 1 of the lab you will:

- Investigate the Wavelet transform and its filter bank implementation for discrete-time signals

- Apply the Haar Discrete Wavelet Transform to compress signals.

Marks will go towards the assessment of EBU6018.

In Part 2 of the lab you will implement a simple JPEG encoder. Marks will go towards the assessment of EBU5303.

✓

The MATLAB programming outcomes from the lab (i.e. MATLAB .m function files and plot figures) are to be handed in as a “folder” of results, showing that you have completed the steps of the lab successfully. A box in the right-hand margin indicates where such an outcome is expected – like this:

Tick the box to indicate that the outcome has been created and placed in the result folder.

You must also answer some questions directly within this document, which must be saved and submitted with other outcomes in your folder of results.

Part 1 (EBU6018)

1 The Wavelet Transform: Introduction

In EBU6018 lectures, we introduced the wavelet transform, defined in terms of the wavelet’s scaling function wavelet function. In those lectures we used Haar Functions as the wavelet function to perform a Discrete Wavelet Transform (DWT).

We also saw that for a discrete-time signal, the Discrete Wavelet Transform (DWT) can be

implemented as a sequence of filtering followed by downsampling. Using this filterbank we do not need to calculate the mother wavelet (t) , we only need the coefficients of the low-pass and high-pass filters H0ωand H1ω.

Various files are provided for this lab.

2 DWT of an input vector s[n].

2.1 Discrete Wavelet Transform Filter Bank

Download the file dwt_haar.m. This implements a Haar Discrete Wavelet Transform using

filtering (convolution) and subsampling.

✓

Explain how this Matlab function works in the text box provided, and check that it implements a normalised Haar DWT using the recursion formulas given in the lecture notes and used in tutorial examples.

Test this function by taking the Haar DWT of the following signal, to 2 steps:

s = [3, 7, -2, -4, 2, 6, 1, -1]

Check the following (and record the values in the text box and save the plots obtained ):

(a) The “length” (Matlab: norm) of the DWT of s, and check it is the same as for s itself.

(b) The first half of the DWT of s contains (scaled) sums of pairs of elements of s, while the

second half contains scaled differences of pairs of elements of s.

(c) The one-step DWT of s is the same as multiplying s (actually the column vector s’) by the following matrix:

✓

W1= 12110000000110001000000100001-1000000001-100110000000000001000-10001-1

Hints: Make sure you always give row vectors to this simple dwt_haar() function.

We are using dyadic scaling so the number of elements in the input vector is a power of 2.

Your answer.

Explanation:

- Line 1, 38:

These two blocks define the entry and exit of the function.

- Line 6 - 11:

N = length(c)-1;  % Max index for filter: 0 .. N

% If no steps to do, or the sequence is a single sample, the DWT is itself

if (0==N | steps == 0)

dwtc = c;

return

end

This block determines the termination condition of the recursion. If the value of steps is 0, it means that the recursion of dwt has ended; specifically, if the length of the input signal is 1, then it has also ended. This judgment is needed in order to terminate at the completion of dwt.

- Line 13 - 17:

% Check that N+1 is divisible by 2

if (mod(N+1,2)~=0)

disp(['Not divisible 2: ' num2str(N+1)]);

return

end

This block checks if the length of the input sequence matches the length that can be handled by this function.

- Line 19 - 25:

% Set the Haar analysis filter

h0 = [1/2  1/2];     % Haar Low-pass filter

h1 = [-1/2 1/2];     % Haar High-pass filter

% Filter the signal

lowpass_c = conv(h0, c);

hipass_c  = conv(h1, c);

This block defines the coefficients of the two filters and applies (convolution) the filter to the original signal c to obtain the low-pass and high-pass signals.

- Line 27 – 29:

% Subsample by factor of 2 and scale

c1 = sqrt(2)*lowpass_c(2:2:end);

d1 = sqrt(2)*hipass_c(2:2:end);

This block down samples the signal to half of its original size and applies the coefficient root number 2.

- Line 31 – 35:

% Recursively call dwt_haar on the low-pass part, with 1 fewer steps

dwtc1 = dwt_haar(c1, steps-1);

% Construct the DWT from c1 and d1

dwtc = [dwtc1 d1];

This recursive block calls the next recursive transformation on c1 and subtracts 1 from the remaining steps value.

The final return value is the merge of the recursive function's return value dwtc1 with d1.

It does implements a normalised Haar DWT using the recursion formulas given in the lecture notes and used in tutorial examples.

Test & Check:

The length of DWT of s is 10.9545, which is equal to itself.

- s = [3, 7, -2, -4, 2, 6, 1, -1]

steps = 2; c1 = [7.0711, -4.2426, 5.6569, 0]; d1 = [-2.8284, 1.4142, -2.8284, 1.4142]

c1[1] = 0.5* 2*(s[1]+s[2]) = 7.0711; c1[2] = 0.5* 2*(s[3]+s[4]) = -4.2426	etc.

d1[1] = 0.5* 2*(s[1]-s[2]) = -2.828 ; d1[2] = 0.5* 2*(s[3]-s[4]) = 1.414	etc.

c1 = [7.0711, -4.2426, 5.6569, 0]

steps = 1; c2 = [2, 4]; d2 = [8, 4]

c2[1] = 0.5* 2*(c1[1]+c1[2]) = 2; c2[2] = 0.5* 2*( c1[3]+ c1[4]) = 4

d2[1] = 0.5* 2*(c1[1]-c1[2]) = 8 ; d2[2] = 0.5* 2*(c1[3]-c1[4]) = 4

dwtc1 = [c2 d2] = [2, 4, 8, 4]

dwt = [dwtc1 d1] = [2, 4, 8, 4, -2.8284, 1.4142, -2.8284, 1.4142]

The above calculation proves the description of Question (b).

It can be considered that multiplying this matrix is equivalent to one step DWT.

2.2 Inverse DWT Filter Bank

Download the file idwt_haar.m , which implements the inverse DWT.

Apply this to the sequencies you transformed in the previous section. Explain what happens in the text box provided.

✓

Explain how this Matlab function works, and check that it implements a recursion formula similar to the synthesis equation given in the lecture slide on the signal recovery filterbank.

Put your explanations in the text box provided and save your plots to confirm that the original sequence is obtained by applying the function idwt_haar.m.

Your answer.

Test code: p122.m

s = [3, 7, -2, -4, 2, 6, 1, -1];

dwts = dwt_haar(s, 2);

idwts = idwt_haar(dwts, 2);

disp(s);

disp(idwts);

Results:

The original sequence is obtained after the inverse conversion.

Explanation:

- Line 1, 57:

These two blocks define the entry and exit of the function.

- Line 6 – 11:

N = length(c)-1;  % Max index for filter: 0 .. N

% If no steps to do, or the sequence is a single sample, the DWT is itself

if (0==N | steps == 0)

idwtc = c;

return

end

This block determines the termination condition of the recursion. If the value of steps is 0, it means that the recursion of idwt has ended; specifically, if the length of the input signal is 1, then it has also ended. This judgment is needed in order to terminate at the completion of idwt.

- Line 13 - 17:

% Check that N+1 is divisible by 2

if (mod(N+1,2)~=0)

disp(['Not divisible 2: ' num2str(N+1)]);

return

end

This block checks if the length of the input sequence matches the length that can be handled by this function.

- Line 19 - 30:

% Set the Haar analysis filters

h0 = [1/2  1/2];     % Haar Low-pass filter

h1 = [-1/2 1/2];     % Haar High-pass filter

% Resynthesis filters are flip (time-reverse) of analysis filters

g0 = h0(end:-1:1);   % This will be [1/2  1/2] for Haar

g1 = h1(end:-1:1);   % This will be [1/2 -1/2] for Haar

% Split signal into the two halves

N2 = (N+1)/2;

c1 = c(1:N2);       % First half of signal

d1 = c(N2+1:end);   % Second half of signal

Redesign the filters in the high-pass and low-pass sections and flip them; split the signal into two parts.

- Line 32 – 33:

% Recursively call idwt_haar on the first half, with 1 fewer steps

idwtc1 = idwt_haar(c1, steps-1);

The idwt_haar function is called recursively to perform the inverse DWT transform on the first half of the signal.

- Line 35 – 44:

% Upsample the signals: first c ...

for i=1:N2

up_c(2*i-1) = idwtc1(i);   % Transfer odd samples

up_c(2*i)   = 0;           % Even samples set to zero

end

% ... then d

for i=1:N2

up_d(2*i-1) = d1(i);

up_d(2*i)   = 0;

end

Double up-sampling of idwtc1 and d1, interpolating zero between them.

- Line 46 – 48

% Filter the signals

filt_c = conv(g0, up_c);

filt_d = conv(g1, up_d);

% Add and scale

idwtc = sqrt(2)*(filt_c + filt_d);

% Answer is defined for n=0..N (in Matlab, 1 to N)

idwtc = idwtc(1:N+1);

The up-sampled signal is applied to the filter, combined and multiplied by the root number 2.

The result is then re-ranged and is the return value of the function, i.e., the sequence after this inverse DWT transform.

3 Signal Compression using the DWT

This exercise is to generate a test signal, plot its DWT, then compress the signal by discarding small values. By reducing the number of non-zero values the result is that fewer bits are needed to represent it. However, the result is some distortion of the original signal.

Download the files: testsig.m, compress.m, uncompress.m, comp_ratio.n.

The file testsig.m generates the following signal:

st= etcos⁡(32πt)0≤t<1/2 0cos⁡(96πt)01/2≤t<3/43/4≤t<7/87/8≤t<1

✓

Modify the file testsig.m so that the first part is now e1.5tcos⁡(32πt)

3.1 Plot this signal and its DWT for 4 levels.

Your answer.

N = 2^16 = 65536

Plot the signal from the modified testsig.m and then its DWT after 1, 2, 3 and then 4 steps of decomposition

3.2 Compress the signal

Applying the file compress.m will compress this signal by transforming it and setting all values below a threshold to zero.

✓

The file uncompress.m inverts the transform carried out by compress.m.

Modify the files compress.m and uncompress.m to include your transform of the signal where indicated.

Your answer.

N = 2^12 = 4096; threshold = 0.42

Plot the original modified signal, the 4-step DWT, the 4-step DWT after thresholding , the reconstructed signal from the inverse DWT and also the error introduced by the DWT thresholding (the difference between the original signal and the reconstructed one). Save the plots obtained.

Part 2 (EBU5303)

4. Image preparation

✓

Create a Matlab function in the file “myJPEG.m”. Read the grayscale image file

mona-lisa.png using the imread matlab function and display it with imshow.

Your answer.

- The size is 250 x 360.

- 250*360*8 = 720000 bits

- int(250/8)*int(360/8) = 31*45 = 1395 blocks

(If the image is filled in order to preserve the full image: ceil(250/8)*ceil(360/8) = 32*45 = 1440 blocks)

What is the dimension of the image (width x hight)? What is its size in bits? Calculate the number of 8x8 blocks of pixels contained in the image.

✓

In myJPEG.m, for one of the channels only and working from left to right, top to bottom, split the image into 8x8 blocks of pixels. Comment your code.

Second Matrix

100    94    85    84    89   103   103    95

88    86    88    84    85    96    99    95

88    86    86    89    93   100   102    98

91    92    90    97    97   105   105   101

90    90    94    97    95   102   105    98

90    91    93    94    92    93    98   100

94    94    89    89    94    98    96    98

97    97    89    89    95    98    94    93

First Matrix

104   103   105    99    92    92    94   100

109   103   105   100    96    92    93    94

109   105    99    98    98   102   105    93

106   110   104    99   103   112   112    97

113   113   112   103   105   109   107    91

121   121   119   107   103    99    95    90

114   117   112   105   105    95    92    93

101   107   103   103   105    96    92    96

Copy and paste the matrices

Print the first two 8x8 matrices of pixel values as examples (use the disp matlab function).

✓

In myJPEG.m, subtract 128 from each pixel values so they range from -128 to 127. Comment your code.

- Print (disp) again the first two 8x8 matrices of pixel values and check they have been “levelled off”.

Second Matrix

-28   -34   -43   -44   -39   -25   -25   -33

-40   -42   -40   -44   -43   -32   -29   -33

-40   -42   -42   -39   -35   -28   -26   -30

-37   -36   -38   -31   -31   -23   -23   -27

-38   -38   -34   -31   -33   -26   -23   -30

-38   -37   -35   -34   -36   -35   -30   -28

-34   -34   -39   -39   -34   -30   -32   -30

-31   -31   -39   -39   -33   -30   -34   -35

First Matrix

-24   -25   -23   -29   -36   -36   -34   -28

-19   -25   -23   -28   -32   -36   -35   -34

-19   -23   -29   -30   -30   -26   -23   -35

-22   -18   -24   -29   -25   -16   -16   -31

-15   -15   -16   -25   -23   -19   -21   -37

-7    -7    -9   -21   -25   -29   -33   -38

-14   -11   -16   -23   -23   -33   -36   -35

-27   -21   -25   -25   -23   -32   -36   -32

Copy and paste the “levelled off” matrices

5. Applying 2D DCT to each block

✓

In myJPEG.m, working from left to right, top to bottom, apply the 2D DCT Matlab function (dct2) to each block of pixel values. Save the results in 8x8 matrices of DCT coefficients values. Comment your code.

- Print (disp) the first two 8x8 matrices of DCT coefficients values.

Second Matrix

-271.5000  -26.0595   10.2119   12.3171   -7.7500    3.2627   -0.1709   -3.2665

-5.0256   -8.1248    4.9878    4.9707   -4.9738    5.2796    1.1810   -0.2532

-8.5391   10.4890    9.6051    7.8289   -1.6004   -4.8424    1.0821    0.4498

2.0890    2.3203    2.9602   -2.8853   -0.6709    2.3292    0.1812    0.4780

11.0000    2.5018   -1.1316    6.4881   -1.7500    0.6649   -1.4254   -2.2534

6.1915   -1.0335    1.9780    4.6095    1.8175   -2.7308   -1.0854   -1.9704

0.2898    0.8183    0.3321    0.2427    2.0159   -0.1773   -1.3551    0.5273

0.5425   -1.4729   -0.5351    3.5111   -0.8709    0.5262    0.2349    0.7409

First Matrix

-201.8750   40.7379    2.5126    3.5727   -9.1250   -0.1836   -1.4467    3.4005

-13.3163  -12.5239    8.7008   -0.1176   -0.4433    7.4667    2.2557   -0.9583

-19.6236    2.9356    0.1402  -14.4638   13.6649   -5.3012    1.9294   -0.8980

8.3138   16.6129    1.0360   -6.8044   -0.6615    0.3763    4.2425    1.4944

-0.1250  -15.7590   -2.1628    1.2498   -4.3750   -0.9280   -0.3218   -1.3851

2.6773   -3.0854    3.0435    0.0228   -2.0441   -1.6778   -1.6912   -1.3525

-0.6661    0.1560    1.4294    0.4091   -0.6541    1.1602   -0.3902   -1.3632

-1.4836   -1.3118    0.0035    2.4986   -0.0045    1.4734   -1.0667    0.5061

Copy and paste the DCT matrices

Paste the matrices

6. Quantisation

✓

In myJPEG.m, create a standard quantisation matrix named Qstd similar to the matrix shown below.

✓

In myJPEG.m, create two more quantisation matrices, one for lower quantisation (Qlow) and one for higher quantisation (Qhigh). Comment your code.

Hint: use quantisation factors to multiply Qstd. The scaled matrices must then be rounded and clipped to have positive integer values ranging from 1 to 255.

Qhigh

32    22    20    32    48    80   102   122

24    24    28    38    52   116   120   110

28    26    32    48    80   114   138   112

28    34    44    58   102   174   160   124

36    44    74   112   136   218   206   154

48    70   110   128   162   208   226   184

98   128   156   174   206   242   240   202

144   184   190   196   224   200   206   198

Qlow

7     5     4     7    10    17    21    26

5     5     6     8    11    24    25    23

6     5     7    10    17    24    29    24

6     7     9    12    21    37    34    26

8     9    16    24    29    46    43    32

10    15    23    27    34    44    47    39

21    27    33    37    43    51    50    42

30    39    40    41    47    42    43    42

Copy and paste the quantisation matrices and explain your choice of quantisation factors.

I chose factor_low = 0.42, because I think the number 42 has a special meaning and is moderately sized as a low quantization factor.

factor_high = 2, because the coefficient is large enough and can keep the values in the quantization matrix within 255 (the maximum value of unsigned int8) to avoid more distortion.

Print (disp) Qlow and Qhigh and explain how you created them (i.e., what quantisation factor you chose and why).

✓

In myJPEG.m, apply quantisation to all the 8x8 matrices of DCT coefficients, using Qstd, Qlow and Qhigh. Comment your code.

Hint: quantisation is achieved by dividing each DCT coefficient by the corresponding element in the quantisation matrix, and then rounding to the nearest integer value.

- Print (disp) the first two 8x8 matrices of quantised DCT coefficients for the three different levels of quantisation (standard, low and high), i.e., 6 matrices in total.

High Quantization 1

-6     2     0     0     0     0     0     0

-1    -1     0     0     0     0     0     0

-1     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

High Quantization 2

-8    -1     1     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Low Quantization 1

-29     8     1     1    -1     0     0     0

-3    -3     1     0     0     0     0     0

-3     1     0    -1     1     0     0     0

1     2     0    -1     0     0     0     0

0    -2     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Low Quantization 2

-39    -5     3     2    -1     0     0     0

-1    -2     1     1     0     0     0     0

-1     2     1     1     0     0     0     0

0     0     0     0     0     0     0     0

1     0     0     0     0     0     0     0

1     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Standard Quantization 2

-17    -2     1     1     0     0     0     0

0    -1     0     0     0     0     0     0

-1     1     1     0     0     0     0     0

0     0     0     0     0     0     0     0

1     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Standard Quantization 1

-13     4     0     0     0     0     0     0

-1    -1     1     0     0     0     0     0

-1     0     0    -1     0     0     0     0

1     1     0     0     0     0     0     0

0    -1     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Copy and paste the six quantised matrices here.

Your answer.

When a lower quantization factor is chosen, larger values within the matrix represent higher accuracy; more non-zero values of the matrix, i.e., more information is retained, and therefore more storage space may be required.

When a higher quantization factor is chosen, smaller values within the matrix represent lower precision; fewer non-zero values of the matrix, i.e., more information is discarded, and therefore less storage space may be required.

When a standard quantization factor is chosen (i.e. factor = 1), the matrix is somewhere in between, i.e. the quality and storage space are relatively balanced.

Compare the 6 matrices and comment.

Your answer.

The higher the value of the quantization matrix, the more effective information in the quantized matrix, the lower the compression rate, and the higher the image quality.

The lower the value of the quantization matrix, the less valid information in the quantized matrix, the higher the compression rate, and the lower the image quality.

Comment on how the choice of quantisation matrix affects the compression rate.

7. Decompression

✓

In myJPEG.m, multiply the quantised DCT values by the corresponding element of the quantisation matrix originally used (i.e., Qstd, Qlow or Qhigh). Comment your code.

Copy and paste the matrices, and comment.

Compared with the matrices in 5b, terms with larger absolute values are retained with partial errors; values with smaller absolute values are discarded as 0.

Matrices with lower quantile coefficients have more accurate values and discard fewer values; matrices with higher quantile coefficients have higher errors in values and discard more values; matrices with standard quantile coefficients are in between.

High Dequantization 2

-256   -22    20     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

High Dequantization 1

-192    44     0     0     0     0     0     0

-24   -24     0     0     0     0     0     0

-28     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Low Dequantization 2

-273   -25    12    14   -10     0     0     0

-5   -10     6     8     0     0     0     0

-6    10     7    10     0     0     0     0

0     0     0     0     0     0     0     0

8     0     0     0     0     0     0     0

10     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Low Dequantization 1

-203    40     4     7   -10     0     0     0

-15   -15     6     0     0     0     0     0

-18     5     0   -10    17     0     0     0

6    14     0   -12     0     0     0     0

0   -18     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Standard Dequantization 1

-208    44     0     0     0     0     0     0

-12   -12    14     0     0     0     0     0

-14     0     0   -24     0     0     0     0

14    17     0     0     0     0     0     0

0   -22     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Standard Dequantization 2

-272   -22    10    16     0     0     0     0

0   -12     0     0     0     0     0     0

-14    13    16     0     0     0     0     0

0     0     0     0     0     0     0     0

18     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

0     0     0     0     0     0     0     0

Print (disp) the first two 8x8 matrices of DCT coefficients for the three different levels of quantisation (standard, low and high), i.e., 6 matrices in total. Compare with the two matrices of question 5b. and comment.

✓

In myJPEG.m, apply the inverse DCT (idct2) to all 8x8 matrices of DCT coefficients (standard, low and high quantisation). Add 128 to each matrix element and round the values. Comment your code.

High IDCT 2

95    94    92    92    93    97   101   103

95    94    92    92    93    97   101   103

95    94    92    92    93    97   101   103

95    94    92    92    93    97   101   103

95    94    92    92    93    97   101   103

95    94    92    92    93    97   101   103

95    94    92    92    93    97   101   103

95    94    92    92    93    97   101   103

High IDCT 1

97    97    96    96    95    94    94    93

101   101   100    99    98    97    96    96

108   107   106   104   103   101   100    99

114   113   111   109   106   104   102   101

118   117   114   111   108   104   102   101

119   117   114   110   106   102    99    97

118   116   113   108   103    99    95    93

117   115   111   106   101    96    92    90

Low IDCT 2

99    92    84    83    93   102   102    96

91    87    82    81    89    98    98    94

90    89    87    87    93   100   101    98

90    93    94    93    97   103   105   103

87    91    92    92    94    99   101   100

89    92    92    91    93    97    98    96

95    95    93    91    94    98    98    95

97    96    92    89    92    97    96    92

Low IDCT 1

103   102   101   100    95    90    93   100

108   105   101    98    96    95    94    93

109   107   100    95   100   106   102    92

106   108   103    97   102   113   110    97

110   115   112   102   102   109   107    96

118   122   118   107   101   100    96    89

117   116   112   107   101    96    92    89

109   104   102   104   103    98    95    95

Standard IDCT 2

98    92    87    87    91    97   100   100

91    87    84    85    90    95    97    97

89    87    86    90    95    99    99    97

92    91    92    97   103   106   105   102

93    92    93    98   103   106   104   101

92    89    88    90    95    97    96    94

96    91    87    86    89    92    93    92

103    97    90    88    90    94    95    95

Standard IDCT 1

103   106   106   100    93    90    96   103

108   107   105    99    93    91    93    95

109   104   100    97    98    98    96    94

109   102    97    99   105   107   103    98

117   110   105   105   108   108   101    94

124   120   115   110   106   100    92    86

113   115   115   110   102    94    89    88

95   102   108   107    99    94    95    98

Copy and paste the matrices, and comment.

Compared with the matrix of 4d, the values of the matrix are closer but with errors; among them, the recovery matrix with low quantization coefficients has smaller errors, the high quantization coefficients have larger errors, and the standard quantization coefficients are in between.

Print (disp) the first two 8x8 matrices of pixels for the three different levels of quantisation (standard, low and high), i.e., 6 matrices in total. Compare with the two matrices of question 4.d and comment.

8. Image reconstruction

✓

In myJPEG.m, reconstruct the images from the 8x8 blocks of pixel values you obtained in question 7 for the three different levels of quantisation (standard, low and high). Comment your code.

✓

Display and save the four images: original, standard, low and high compression.

- Compare the images and comment. You should clearly see blocky artefacts in the high compression image. If not, go back to Question 6 and modify the Qhigh matrix.

Your comments.

Hint: You may have to cast your pixel values to uint8 to display the images.

Comparing the three compressed images with the original image, we can see that the higher the quantization factor, the less detail is retained in the image; the image with high quantization factor has obvious blocking artifacts; the image with high quantization factor has obvious ringing artifacts at the edges; the image with low quantization factor is closest to the original image; the image with standard quantization factor is in between.

Handing In

Compile the answers to the exercises, including the answers to specific questions, program listings (including comments), and plots from experiments, into a “folder” of results showing that you have completed the lab.

Name the folder: EBU6018_5303_Lab2

Rename this document ‘Lab2_xxxxxxxx’ where xxxxxxxx is your QM student number and save it in your result folder together with your MATLAB files and plot images.

Submit the folder as a zip archive on QMplus in BOTH the EBU6018 and EBU5303 course areas before the deadline (i.e., submit twice the same zip archive).

IMPORTANT: Plagiarism (copying from other students or copying the work of others without proper referencing) is cheating, and will not be tolerated.

IF TWO “FOLDERS” ARE FOUND TO CONTAIN IDENTICAL MATERIAL, BOTH WILL BE GIVEN A MARK OF ZERO.

Marking scheme

Part 1 EBU6018 (max 45 marks):

Q2.1 : up to 16 marks

Explanations up to 5 marks

Plots up to 6 marks

Calculations up to 5 marks

Q2.2 : up to 9 marks

Plots up to 6 marks

Explanations up to 3 marks

Q3.1 : up to 10 marks for plots.

Q3.2 : up to 10 marks for plots

Part 2 EBU5303 (max 80 marks):

Q4. Image Preparation: up to 15 marks

Q4.b up to 3 marks

Q4.c up to 4 marks

Q4.d up to 4 marks (2 per matrix)

Q4.e up to 2 marks

Q4.f up to 2 marks (1 per matrix)

Q5. 2D DCT: up to 8 marks

Q5.a up to 4 marks

Q5.b up to 4 marks (2 per matrix)

Q6. Quantisation: up to 27 marks

Q6.a up to 1 mark

Q6.b up to 5 marks

Q6.c up to 5 marks (1 per matrix, and 3 for the explanations)

Q6.d up to 5 marks

Q6.e up to 3 marks (0.5 per matrix)

Q6.f up to 5 marks

Q6.g up to 3 marks

Q7. Decompression: up to 18 marks

Q7.a up to 3 marks

Q7.b up to 5 marks

Q7.c up to 6 marks

Q7.d up to 4 marks

Q8. Reconstruction: up to 12 marks

Q8.a up to 5 marks

Q8.b up to 3 marks (1 per image)

Q8.c up to 4 marks

Updated by MPD, MEPD

Modified ARW for EBU6018.

Modified MLB for EBU5303.